| DroneSMS Job Hazard Analysis (JHA) Pillar 2: Risk Management ¦ Drone Cleaning Operations | Risk Methodology MIL-STD-882E w/Change 1 DoD System Safety Standard 27 September 2023 Adapted for commercial drone cleaning operations. Severity dollar thresholds adjusted for small commercial scope. |
| --- | --- |

| 1 | Job Information |
| --- | --- |

| Operator / Company |  | JHA Number |  |
| --- | --- | --- | --- |
| Remote Pilot in Command |  | Date Prepared |  |
| Client / Property Owner |  | Job Date |  |
| Site Address |  | Drone Platform |  |
| Job Type / Scope |  | Crew Members |  |
| Weather Conditions |  | FAA Airspace Class |  |

| 2 | Risk Assessment Methodology — Adapted from MIL-STD-882E |
| --- | --- |

The following severity and probability definitions are adapted from MIL-STD-882E w/Change 1, DoD System Safety Standard Practice (27 September 2023). Dollar thresholds have been adjusted for small commercial drone cleaning operations. The Risk Assessment Matrix (RAC) structure is preserved from the original standard.

TABLE I — HAZARD SEVERITY CATEGORIES

| Description | Category | Drone Cleaning Application | Mishap Result Criteria (Adapted from MIL-STD-882E Table I) |
| --- | --- | --- | --- |
| Catastrophic | 1 | Drone strikes person, fall zone failure with injury, major chemical release, structural property damage | Could result in one or more of the following: death, permanent total disability, irreversible significant environmental impact, or monetary loss exceeding $100,000. (Original MIL-STD-882E threshold: $10M) |
| Critical | 2 | Pressure equipment failure, chemical exposure requiring medical care, multiple personnel injuries, significant property damage | Could result in one or more of the following: permanent partial disability, injuries or occupational illness resulting in hospitalization of at least one person, reversible significant environmental impact, or monetary loss equal to or exceeding $10,000 but less than $100,000. (Original: $1M–$10M) |
| Marginal | 3 | Hose trip hazard causing injury, minor chemical skin contact, slip/fall resulting in lost work day, moderate equipment damage | Could result in one or more of the following: injury or occupational illness resulting in one or more lost work days, reversible moderate environmental impact, or monetary loss equal to or exceeding $1,000 but less than $10,000. (Original: $100K–$1M) |
| Negligible | 4 | Minor equipment scratch, small water spill contained on site, minor PPE issue with no injury | Could result in one or more of the following: injury or occupational illness not resulting in a lost work day, minimal environmental impact, or monetary loss less than $1,000. (Original: <$100K) |

TABLE II — PROBABILITY LEVELS

| Description | Level | Likelihood (Single Job) | Likelihood (Fleet / Over Time) |
| --- | --- | --- | --- |
| Frequent | A | Likely to occur on this job. | Continuously experienced across operations. |
| Probable | B | Will likely occur sometime during this job. | Will occur frequently across operations. |
| Occasional | C | Could occur during this job or similar jobs. | Will occur several times across operations. |
| Remote | D | Unlikely but possible on this job. | Unlikely but can reasonably be expected to occur across operations. |
| Improbable | E | So unlikely it may not occur on this job or similar jobs. | Unlikely to occur, but possible across all operations. |
| Eliminated | F | Hazard has been identified and fully eliminated through controls. | Incapable of occurrence. Hazard is removed. |

TABLE III — RISK ASSESSMENT MATRIX (RAC)

The Risk Assessment Code (RAC) is the combination of one Severity Category (1–4) and one Probability Level (A–F). Example: RAC 1A = Catastrophic severity + Frequent probability = HIGH risk. Use this matrix to determine the initial risk level for each identified hazard.

| SEVERITY → PROBABILITY ↓ | Catastrophic (1) | Critical (2) | Marginal (3) | Negligible (4) |
| --- | --- | --- | --- | --- |
| Frequent (A) | HIGH | HIGH | SERIOUS | MEDIUM |
| Probable (B) | HIGH | HIGH | SERIOUS | MEDIUM |
| Occasional (C) | HIGH | SERIOUS | MEDIUM | LOW |
| Remote (D) | SERIOUS | MEDIUM | MEDIUM | LOW |
| Improbable (E) | MEDIUM | MEDIUM | MEDIUM | LOW |
| Eliminated (F) | ELIMINATED — Hazard identified and fully removed through design or control measures |  |  |  |

RISK LEVEL DEFINITIONS

| HIGH | Unacceptable. Job shall not proceed until risk is reduced. Requires immediate mitigation, redesign of operation, or cancellation. RPIC must document specific controls and obtain sign-off before any operations begin. |
| --- | --- |
| SERIOUS | Undesirable. Proceed only with specific documented controls in place. RPIC must brief all crew on this hazard specifically. Residual risk must be reduced to Medium or Low before operations begin. |
| MEDIUM | Acceptable with controls. Document controls and verify implementation before operations. Crew briefing required. RPIC monitors throughout job. |
| LOW | Acceptable. Standard operating procedures apply. Document and monitor. No additional controls required beyond standard PPE and operating procedures. |

| 3 | Hazard Mitigation — Order of Precedence (MIL-STD-882E 4.3.4) |
| --- | --- |

Apply risk mitigation in the following order of decreasing effectiveness. Always attempt to eliminate the hazard first before moving to lower-precedence controls.

| Priority | Control Method | Application to Drone Cleaning Operations |
| --- | --- | --- |
| 1 | Eliminate by Design | Change the operation to remove the hazard entirely. Example: choose a drone cleaning method that does not require chemical application, eliminating chemical exposure hazard. |
| 2 | Reduce by Design | Alter the operation to reduce severity or probability. Example: reduce spray pressure to lower equipment failure risk; adjust flight path to reduce public exposure. |
| 3 | Engineered Controls | Use physical barriers or devices to interrupt the hazard sequence. Example: install water containment berm, use pressure relief valve, establish physical exclusion barrier around work zone. |
| 4 | Warning Devices | Alert personnel to hazardous conditions. Example: signage on exclusion perimeter, audible alarm on pressure system, visual observer positioned to alert RPIC. |
| 5 | Procedures, Training, PPE | Use signage, written procedures, crew training, and PPE as last line of defense. Note: Per MIL-STD-882E 4.3.4e, PPE and procedures alone shall not be the only control for Catastrophic or Critical hazards. |

| 4 | Hazard Identification and Risk Assessment |
| --- | --- |

Instructions: For each identified hazard, assign a Severity Category (1–4) and Probability Level (A–F) using Tables I and II. Determine the RAC and initial Risk Level using Table III. Document controls and reassess residual risk after controls are applied. All HIGH and SERIOUS hazards must be reduced before operations begin.

| # | Hazard Description | Severity (1–4) | Prob (A–F) | RAC | Initial Risk | Controls / Mitigation Measures | Residual Risk | Notes / Owner |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Drone fly-away or loss of control over public / personnel | 1 | D | 1D | Serious | Pre-flight checks, VLOS maintained, VO assigned, exclusion zone established, emergency procedure briefed | Medium | RPIC |
| 2 | Drone rotor strike to ground personnel or public | 1 | D | 1D | Serious | Exclusion zone marked and enforced, VO monitors perimeter, public warning signage posted, job site access controlled | Low | VO |
| 3 | Pressure equipment failure / hose burst | 2 | C | 2C | Serious | Pre-job equipment inspection, pressure relief valve verified, crew briefed on hose hazard, PPE worn by all crew | Medium | RPIC |
| 4 | Hose trip hazard to crew or public | 3 | B | 3B | Serious | Hoses routed and secured, cones or barriers placed, ground crew maintains awareness, public exclusion enforced | Low | Crew |
| 5 | Chemical / soft wash exposure to personnel | 2 | C | 2C | Serious | SDS reviewed, appropriate PPE issued, dilution ratios verified, chemical containment plan in place, eyewash available | Medium | RPIC |
| 6 | Chemical runoff to storm drain or waterway | 2 | C | 2C | Serious | Storm drain covered or blocked, containment berm deployed where required, runoff direction assessed, municipal requirements checked | Medium | RPIC |
| 7 | Water / slip hazard on ground surface | 3 | A | 3A | Serious | Non-slip footwear required, wet surface signage posted, work area monitored continuously, crew briefed | Low | Crew |
| 8 | Property damage from drone impact or water intrusion | 3 | C | 3C | Medium | Pre-job site photo documentation, property access agreement signed, careful flight path planning, post-job documentation | Low | RPIC |
| 9 | Electrical hazard from overhead power lines | 1 | D | 1D | Serious | Site survey identifies all power lines, minimum clearance distances maintained, VO monitors proximity, operations suspended if clearance cannot be maintained | Medium | RPIC |
| 10 | Battery fire or thermal runaway (LiPo) | 2 | D | 2D | Medium | Battery inspection pre-flight, batteries stored in LiPo-safe bags during transport, fire extinguisher on site, no charging near combustibles | Low | RPIC |
| 11 | Unauthorized airspace / LAANC violation | 2 | D | 2D | Medium | Airspace checked via app, LAANC authorization obtained where required, authorization documentation in job file | Low | RPIC |
| 12 | Heat stress / dehydration (crew) | 3 | B | 3B | Serious | Water on site, rest breaks scheduled, heat index monitored, work suspended if conditions exceed safe limits | Low | RPIC |
| 13 |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  |

| 5 | Overall Job Risk Determination |
| --- | --- |

| Highest Initial Risk Level | Number of HIGH/SERIOUS Hazards | Highest Residual Risk After Controls | Overall Job Risk Rating |
| --- | --- | --- | --- |

| 6 | PPE Requirements |
| --- | --- |

| Safety Glasses | Chemical Resistant Gloves | Non-Slip Footwear | High-Vis Vest | Hearing Protection | Other (specify) |
| --- | --- | --- | --- | --- | --- |
| Required: Y / N | Required: Y / N | Required: Y / N | Required: Y / N | Required: Y / N |  |

| 7 | Emergency Procedures and Contacts |
| --- | --- |

| Nearest Hospital |  | Emergency Contact |  |
| --- | --- | --- | --- |
| Stop-Work Authority | Any crew member may call stop-work without penalty. | Drone Incident Procedure | Land immediately, secure area, assess injuries, notify RPIC, document, report per FAA requirements if applicable. |

| 8 | Crew Briefing Acknowledgment |
| --- | --- |

All crew members must sign below confirming they have been briefed on identified hazards, required controls, PPE requirements, emergency procedures, and stop-work authority. This document is part of the job compliance record stored in DroneSMS.

| Printed Name | Role | Signature | Date |
| --- | --- | --- | --- |

| 9 | RPIC Certification |
| --- | --- |

I certify that this Job Hazard Analysis has been completed in good faith prior to operations, that all identified hazards have been assessed using the MIL-STD-882E adapted methodology, that controls are in place before work begins, and that all crew members have been briefed on this JHA. I understand that as RPIC I retain stop-work authority and responsibility for the safety of this operation.

| RPIC Printed Name | RPIC Signature | Date / Time |
| --- | --- | --- |